PARALLEL & DISTRIBUTED COMPUTING

# 1.1 Why Parallel and Distributed Computing?

A modern computer is expected to process enormous amounts of data quickly. Examples include AI model training, video processing, scientific simulation, online services, cybersecurity monitoring, financial transactions, and large database operations. A sequential program that gives all of the work to one processor can become too slow, so computing systems increasingly divide work across multiple processing resources.

## 1.1.1 Limits of Single-Core Performance

For many years, computer performance improved primarily by making one processor core faster. Designers increased clock frequency, improved pipelines, caches, branch prediction, instruction execution, and other microarchitectural features. However, each improvement faced physical or architectural limits.

Simple analogy Imagine one worker processing 10,000 documents. Even if the worker becomes faster, there is a practical limit to how fast one person can work. A different strategy is to divide the documents among several workers. Parallel computing applies the same basic idea to computation.

### Clock-frequency limit

Increasing clock frequency means circuits switch more often. Beyond a practical point, higher frequency increases power consumption and heat, making further increases difficult.

### Memory bottleneck

A processor may be able to execute instructions faster than the memory system can supply the required data. The processor then spends time waiting rather than computing.

### Diminishing returns

Making one core faster does not guarantee the whole application becomes equally faster because I/O, memory access, synchronization, and serial sections may remain bottlenecks.

### Physical limits

Transistor behavior, voltage, current, leakage, heat removal, and manufacturing constraints all place limits on continuous single-core scaling.

## 1.1.2 Moore’s Law

Moore’s Law is a historical observation associated with Gordon Moore: the number of transistors that could be integrated on a chip tended to grow rapidly over time, commonly summarized as roughly doubling over a period of about 18–24 months in its classic formulation. It is an industry trend, not a literal physical law.

More transistors gave designers more resources for larger caches, more complex control logic, vector units, specialized accelerators, and—very importantly—multiple processing cores.

Memory aid Moore → MORE transistors on a chip. More transistors increased the design possibilities for higher computing capability.

## 1.1.3 Dennard Scaling

Dennard scaling describes a historical relationship in which shrinking transistors allowed voltage and other electrical parameters to scale down, helping engineers keep power density manageable while increasing transistor density and performance. For a long time, this meant smaller transistors could provide more performance without a proportional explosion in power.

Classical Dennard scaling eventually broke down because voltage and power could not continue scaling downward at the same rate. As a result, simply making transistors smaller no longer automatically produced the same easy gains in frequency and energy efficiency.

## 1.1.4 Moore’s Law vs. Dennard Scaling

| Aspect | Moore’s Law | Dennard Scaling |
| --- | --- | --- |
| Main idea | Rapid growth in transistor density | Historical scaling of transistor dimensions, voltage, and power density |
| What it enabled | More computational resources on chips | More performance with historically manageable power density |
| Key limitation | Economic/manufacturing difficulty as scaling continues | Classical relationship largely broke down |
| Connection to multicore | Extra transistors can be used for multiple cores | Power limits encouraged alternatives to endlessly raising frequency |

## 1.1.5 The Multicore Era

Instead of trying to make one core dramatically faster, processor designers increasingly placed multiple cores on the same chip. Each core can execute instructions independently, allowing software to run multiple threads or processes concurrently and, when hardware resources permit, in parallel.

Multicore CPU +------------------------------+ ¦ Core 1 ¦ Core 2 ¦ Core 3 ¦ Core 4 ¦ +------------------------------+ Task A Task B Task C Task D

The benefit is not automatic. A program must contain enough work that can be executed simultaneously. If most of the program is inherently sequential, adding more cores may provide limited speedup.

## 1.1.6 Power Wall

The power wall is the practical limit on how much power a processor can consume because electrical power becomes heat and the system must remove that heat. Raising clock frequency and voltage can increase power significantly, so there is a point where additional frequency is no longer practical.

Higher frequency → higher power → more heat → harder cooling → thermal/power limit

## 1.1.7 Thermal Constraints

A thermal constraint means the computing device must operate within a safe temperature range. If a processor becomes too hot, the system may reduce its frequency or voltage, a process commonly called thermal throttling. Sustained high temperature can also affect reliability and energy efficiency.

Why this matters A desktop server can use large heat sinks, fans, or liquid cooling. A smartphone has a small battery and limited physical space, so it must achieve high performance under much tighter energy and thermal constraints.

## 1.1.8 Why Parallel and Distributed Computing?

A single core cannot scale in speed forever.

Multiple cores allow several parts of a workload to execute concurrently.

Vector/SIMD units process multiple data elements with one instruction.

Distributed systems allow many networked computers to cooperate.

Cloud and large-scale data platforms can use clusters of servers to process workloads that are too large for one machine.

# 1.2 Fundamental Concepts

Before studying advanced parallel algorithms, students must clearly distinguish several concepts that are often confused: concurrency, parallelism, distribution, and granularity.

## 1.2.1 Concurrency vs. Parallelism

Concurrency means that multiple tasks can make progress during the same overall period. It does not necessarily mean that the tasks are executing at exactly the same instant. A single CPU core can support concurrency by rapidly switching between tasks.

Parallelism means that multiple tasks are actually executing simultaneously using multiple processing resources, such as multiple CPU cores or multiple GPU execution units.

Concurrency on one core: Task A → Task B → Task A → Task C → Task B Parallelism on multiple cores: Core 1 → Task A Core 2 → Task B Core 3 → Task C

| Feature | Concurrency | Parallelism | Easy memory aid |
| --- | --- | --- | --- |
| Core requirement | Can happen on one core | Usually needs multiple execution resources | Concurrency ≠ simultaneous execution |
| Main concern | Managing multiple tasks | Executing multiple tasks at once | Parallel = simultaneous |
| Example | Multitasking on one CPU | Matrix calculation on 8 cores | Many workers at once |

## 1.2.2 Parallelism vs. Distribution

Parallelism focuses on simultaneous execution. Distribution focuses on how the computation is spread across independent machines. A distributed system normally contains multiple computers connected by a network, each with its own processor, memory, operating system, and local resources.

Distributed system Computer A ---- network ---- Computer B ¦ ¦ Computer C ---------------- Computer D

A distributed application may also be parallel. For example, each server in a cluster may use multiple CPU cores. Thus, modern systems often combine distribution across machines with parallelism inside each machine.

| Aspect | Parallel Computing | Distributed Computing |
| --- | --- | --- |
| Main idea | Execute multiple tasks at the same time | Coordinate multiple independent computers |
| Communication | Shared memory or high-speed interconnects are common | Network communication is central |
| Memory model | Often shared or tightly coupled | Usually separate memory per machine |
| Example | Multicore matrix processing | Web/data-processing cluster |

## 1.2.3 Granularity: Fine-Grained vs. Coarse-Grained

Granularity describes the size of the tasks into which a problem is divided. The two broad categories are fine-grained and coarse-grained parallelism.

### Fine-Grained Parallelism

Fine-grained parallelism divides work into many small tasks. Because there are many task boundaries, the system may need frequent scheduling, synchronization, and communication.

Advantage: many opportunities for parallel execution.

Advantage: often easier to balance work when task sizes vary.

Disadvantage: higher coordination and synchronization overhead.

Risk: if each task is extremely small, overhead can exceed useful computation.

### Coarse-Grained Parallelism

Coarse-grained parallelism divides the problem into fewer, larger tasks. Each task does substantial computation before the worker communicates or synchronizes.

Advantage: lower scheduling and communication overhead.

Advantage: less frequent synchronization.

Disadvantage: harder to balance work if some tasks take much longer than others.

Real-life analogy Fine-grained: one worker cleans one chair. There are many tiny tasks and a lot of coordination. Coarse-grained: one worker cleans one floor. There is less coordination, but one floor may take much longer than another.

| Feature | Fine-Grained | Coarse-Grained |
| --- | --- | --- |
| Task size | Small | Large |
| Number of tasks | Many | Fewer |
| Communication | Often higher | Usually lower |
| Scheduling overhead | Higher | Lower |
| Load balancing | Can be easier | Can be harder |

# 1.3 Flynn’s Taxonomy

Flynn’s Taxonomy is a classical classification of computer architectures based on the number of instruction streams and data streams being processed.

The key to the taxonomy S = Single, M = Multiple, I = Instruction, D = Data. The first letter describes the number of instruction streams; the second describes the number of data streams.

## 1.3.1 SISD — Single Instruction, Single Data

SISD represents the traditional sequential execution model: one instruction stream operates on one data stream. It is the simplest of the four categories.

Instruction stream ¦ v [ CPU ] ¦ v Data stream

A simple single-core sequential program can be viewed conceptually as SISD. Modern processors are more complicated internally, so this should be treated as a textbook classification rather than a complete description of every CPU.

Real-world example: a simple sequential embedded application executing one instruction sequence at a time on a single processing unit.

## 1.3.2 SIMD — Single Instruction, Multiple Data

SIMD applies the same instruction to multiple data elements in parallel. This is especially useful when the same operation must be repeated over large arrays, image pixels, audio samples, or vector data.

One vector instruction ¦ ----------------- ¦ ¦ ¦ ¦ ¦ Data1 Data2 Data3 Data4 ¦ ¦ ¦ ¦ ¦ Result1 Result2 Result3 Result4

For example, adding two vectors [1,2,3,4] and [10,20,30,40] gives [11,22,33,44]. A SIMD instruction can conceptually process several elements in one operation instead of executing four separate scalar operations.

Real-world examples include CPU vector instruction sets such as x86 SSE/AVX and ARM NEON. Image and multimedia processing are common use cases.

## 1.3.3 MISD — Multiple Instruction, Single Data

MISD means that multiple instruction streams operate on the same data stream. It is relatively uncommon in general-purpose computing.

Same input data ¦ ----------------------- ¦ ¦ ¦ Algorithm A Algorithm B Algorithm C

A useful conceptual example is a fault-tolerant system in which the same sensor input is processed independently by different algorithms or implementations and the outputs are compared to detect errors. Not every system of this type is strictly MISD, but the pattern helps students understand why MISD exists in the taxonomy.

## 1.3.4 MIMD — Multiple Instruction, Multiple Data

MIMD allows multiple processing elements to execute different instructions on different data at the same time. This model is strongly associated with modern multicore and multiprocessor general-purpose systems.

Core 1 → Instruction A → Data A Core 2 → Instruction B → Data B Core 3 → Instruction C → Data C Core 4 → Instruction D → Data D

Real-world examples include multicore servers handling different threads, scientific applications with different tasks, and web servers processing different client requests concurrently.

## 1.3.5 Flynn’s Taxonomy at a Glance

| Category | Instruction Streams | Data Streams | Core Idea | Common Example |
| --- | --- | --- | --- | --- |
| SISD | Single | Single | One instruction sequence processes one data stream | Sequential program |
| SIMD | Single | Multiple | Same operation across many values | Image/vector processing |
| MISD | Multiple | Single | Different operations on one data stream | Fault-tolerant conceptual patterns |
| MIMD | Multiple | Multiple | Different operations on different data | Multicore/server systems |

## 1.3.6 One Problem, Multiple Levels of Parallelism

Suppose a company must analyze one million images. A single-core sequential design processes images one after another. A multicore design can distribute image ranges among CPU cores. Inside each core, SIMD instructions can process multiple pixel values together. If the dataset is too large for one machine, the company can distribute image batches across several servers.

Large image-analysis workload ¦ v Multiple servers (distribution) ¦ v Multiple CPU cores (MIMD-style parallelism) ¦ v SIMD/vector operations on pixel groups ¦ v Individual pixels / data elements

This illustrates an important modern computing principle: several levels of parallelism can coexist in one real system.